_________________________________________________________________________________________________________________________________

DataFlash® Error Correction

Reference

Version 1.0

Copyright  1999 – 2002  Atmel Corporation  All Rights Reserved

2125 Orchard Parkway

San Jose, CA 95131

Phone (408) 441-0311

Table of Contents

Chapter 1: Introduction	1

Chapter 2: Using Error Correction	2

Starting	2

Example	2

Chapter 3: Error Correction Function Calls	3

correct_errors	4

decode_data	5

encode_data	6

Appendix A: Memory Structures	7

Exponent table array	7

Log table array	7

generated polynomial array	7

Chapter

1

Chapter 1: Introduction

Reed-Solomon codes are block-based error correcting codes with a wide range of applications in digital communications and storage. Reed-Solomon codes are used to correct errors in many systems including:

- Storage devices (including tape, Compact Disk, DVD, barcodes, file systems etc)

- Wireless or mobile communications (including cellular telephones, microwave links, etc)

- Satellite communications

The Reed-Solomon encoder takes a block of digital data and adds extra "redundant" bits. Errors occur during transmission or storage for a number of reasons (for example noise or interference, scratches on a CD, etc). The Reed-Solomon decoder processes each block and attempts to correct errors and recover the original data. The number and type of errors that can be corrected depends on the characteristics of the Reed-Solomon code. For this application 2 bytes out of each 32 bytes can be corrected.

Reed Solomon codes are a subset of BCH codes and are linear block codes. A Reed-Solomon code is specified as RS(n,k) with s-bit symbols.

This means that the encoder takes k data symbols of s bits each and adds parity symbols to make an n symbol codeword. There are n-k parity symbols of s bits each. A Reed-Solomon decoder can correct up to t symbols that contain errors in a codeword, where 2t = n-k.

The following diagram shows a typical Reed-Solomon codeword (this is known as a Systematic code because the data is left unchanged and the parity symbols are appended):

This version is a RS(32,28) with 8 bit symbols.

Chapter 2: Using Error Correction

Chapter

2

## Starting

There are three tables used during error encoding, decoding and correction. These tables need to be initialized for the error correction to work correctly. To initialize these tables call the function

initialize_ecc();

The function encode_data(data, size, codeword); requires a pointer to the data that is to be encoded, the size of the data (less than or equal to 28) and a pointer to a 32 byte codeword array where the encoded data and the 4 extra parity bytes are placed.

The function decode_data(codeword, syndrome); requires a pointer to a 32 byte codeword and a pointer to an eight byte syndrome array.  The decode function will determine if there is an error. If so then it will return a non-zero value. The function will also fill out the syndrome array for later use in the correct_errors(codeword, syndrome); function.

The function correct_errors(codeword, syndrome); requires a pointer to a 32 byte codeword and a pointer to the eight byte syndrome array filled out with the decode_data function. This function is capable of correcting up to 2 bad bytes out of each 32 byte codeword.

## Example

unsigned char codeword[32];

unsigned char data[28];

unsigned char syndrome[SYNDROME_LENGTH];

encode_data(data, 28, codeword);

status = decode_data(codeword, syndrome);

if (status != 0)

{

status = correct_errors(codeword, syndrome);

if (status) ASSERT

}

Chapter 3: Error Correction Function Calls

Chapter

3

This chapter describes all of the API function calls the RS(32,28) error correction software.

# correct_errors

Description:	This function requires a pointer to a 32 byte codeword and a pointer to the eight byte syndrome array filled out with the decode_data function. This function is capable of correcting up to 2 bad bytes out of each 32 byte codeword..

Parameters:

| Byte codeword | This is the pointer to the 32 byte codeword to be corrected. |
| --- | --- |
| Byte syndrome | This is the pointer to the 8 byte array passed into decode_data. |

Return:

| int | Returns a 0 if the correction was successful. Returns a 1 if there were more errors than could be corrected. |
| --- | --- |

Prototype:	int correct_errors(byte codeword[], byte syndrome[])

Example:

unsigned char codeword[32];

unsigned char syndrome[SYNDROME_LENGTH]; // 8 bytes

status = correct_errors(codeword, syndrome);

# decode_data

Description:	This function requires a pointer to a 32 byte codeword and a pointer to an eight byte syndrome. This function will determine if there are any errors in the codeword. This function does not actually correct any errors. It only detects them.

Parameters:

| Byte codeword | This is the pointer to the 32 byte codeword to be corrected. |
| --- | --- |
| Byte syndrome | This is the syndrome pointer. This is used to correct any errors in correct_errors |

Return:

| int | Returns a 0 if there are not any errors. A 1 is returned if errors are detected. |
| --- | --- |

Prototype:	int decode_data(byte codeword[], byte syndrome[])

Example:

unsigned char codeword[32];

unsigned char syndrome[SYNDROME_LENGTH]; // 8 bytes

status = decode_data (codeword, syndrome);

# encode_data

Description:	This function requires a pointer to the data that is to be encoded, the size of the data (less than or equal to 28) and a pointer to a 32 byte codeword array where the encoded data and the 4 extra parity bytes are placed.

.

Parameters:

| Byte * data | This is the pointer to the 28 byte data to be error encoded. |
| --- | --- |
| Byte * codeword | This is the pointer to the 32 byte codeword array where the new codeword will be placedto be corrected. |
| Byte size | This is the size of the data to be encoded. It may be from 1 to 28. |

Return:

| None |  |
| --- | --- |

Prototype:	int encode_data(byte codeword[], byte syndrome[])

Example:

unsigned char codeword[32];

unsigned char data[28];

unsigned char syndrome[SYNDROME_LENGTH]; // 8 bytes

encode_data(data, 28, codeword);

Appendix A: Memory Structures

Appendix

A

Appendix

A

# Exponent table array

byte exp_table[512];

# Log table array

byte log_table[256];

# Generated polynomial array

#define DOUBLE_POLY_SIZE 16

byte gen_poly[DOUBLE_POLY_SIZE];

Index

C

correct_errors, 4

D

decode_data, 5

E

encode_data, 6

Example, 2

Exponent table array, 7

G

Generated polynomial array, 7

I

Introdcution, 1

L

Log table array, 7

S

Starting, 2

U

Using Error Correction, 2