_________________________________________________________________________________________________________________________________

DataFlash® File System

Reference Manual

Version 1.1

Copyright  1999 – 2003  Atmel Corporation  All Rights Reserved

2125 Orchard Parkway

San Jose, CA 95131

Phone (408) 441-0311

Table of Contents

Chapter 1 Introduction	4

Overview of FAT File System	4

Boot Sector and BPB	4

The FAT	5

Root Directory	5

Data	5

Non-Supported Features	5

Partitioning	6

FAT32	6

Long Filenames	6

Flash Reliability Enhancements	6

Run-Time Errors	6

Data Caching	7

Wear Leveling	7

Error Correction	8

Chapter 2 Product Perspective	9

Overview of the Model	9

Device Model	10

Driver Object	10

Device Object	11

Dispatch Interface	12

IO_DISPATCH Method	12

Parameters:	12

Return status:	12

Prototype:	12

Example:	13

I/O Packet	13

Device Driver Table	14

Device Driver Identification Section	14

Driver Table	14

DISCLAIMER	15

Chapter 3 System Requirements	16

Platform Compatibility	16

Memory Management	16

Character Functions	16

String Management	17

Time Management	17

Synchronization Objects	18

C Language Cross Reference	19

Memory Management	19

Character Functions	19

String Management	20

Time Management	20

Synchronization	21

Memory Requirements	21

Chapter 4 File System API	22

File System Methods	22

Volume Specific Operations	22

Directory Specific Operations	23

File  Specific Operations	23

Miscellaneous (Support) Operations	23

Structures	24

DIRECT	24

STAT	24

Types	25

System Programmer’s Interface	26

sys_close()	26

sys_closedir()	27

sys_creat()	28

sys_devstats()	29

sys_eof()	30

sys_format()	31

sys_lseek()	32

sys_mkdir()	33

sys_mount()	34

sys_open()	35

sys_opendir()	37

sys_read()	38

sys_readdir()	39

sys_remove()	40

sys_rename()	41

sys_rewinddir()	42

sys_rmdir()	43

sys_stat()	44

sys_tell()	45

sys_umount()	46

sys_unlink()	47

sys_write()	48

Chapter 5 I/O Manager	49

I/O Table	49

IOEntry	49

Initialization	50

File Control Block	51

Implementation	51

Chapter 6 File System Driver	53

FSD Objects	53

FAT Device Extension	53

File Object	54

FAT File Extension	54

Dispatch Operations	55

DEVICE_IO_OPEN	55

DEVICE_IO_CLOSE	55

DEVICE_IO_READ	56

DEVICE_IO_WRITE	56

DEVICE_IO_CONTROL	56

mount	56

unmount	57

format	57

lseek	57

rename	57

mkdir	57

rmdir	58

readdir	58

filestat	58

Chapter 7 Integration and Usage Tips	59

ROOT Directory Management	59

FAT Determination	59

Initialization	60

Appendix A Miscellaneous	61

Compatible errno codes	61

DEVSTATS	63

BLOCK_DEVSTATS	63

CHAR_DEVSTATS	64

Exception Handling	64

FUNCTIONALITY	64

DEFINED EXCEPTIONS	65

Example	65

Appendix B Memory Requirements	66

Appendix C Examples	68

Doing a UNIX “ls” (directory listing)	68

localtime	72

Index	76

Chapter 1
Introduction

The Atmel DataFlash File System is a FAT12/FAT16 Design and targets Atmel Flash EEPROM devices. It delivers a layered driver architecture using a simple binding model allowing for OS-agnostic portability.

# Overview of FAT File System

Developed by Microsoft in the early 1980s, FAT, File Allocation Table, is a file system used to keep track of file fragments on a storage media. These fragments are called file clusters and a file is composed of one or more of these clusters up to a given maximum (typically 2 Gigabytes). Each cluster is composed of one or more smaller segments called sectors, which generally adhere to the default write block size of the part. The Atmel driver defaults to 512 bytes since most applications worldwide hardwire this value internally.

The Volume is divided up into 4 regions: Boot, FAT, Root, and Data.

## Boot Sector and BPB

Located at the first sector of the file volume. It does not necessarily mean that it exists at the very front of the device, since this portion of the device may be reserved for other use. The Boot Sector contains Metadata that is important in determining the size and type of the volume, i.e. FAT12 or FAT16 (FAT32 is not supported, as will be explained shortly).

If at anytime the Boot Sector becomes corrupted, then the file system is rendered useless and all data shall be lost.

## The FAT

The next data structure, immediately following the Boot Sector, is the File Allocation Table or FAT as it is better known. The FAT defines all of the clusters within the Data Region of the file system. Each entry in the table is an index to a corresponding cluster and the contents of that entry specify whether the cluster is free, or is an end of file, or a link to the next cluster of the file or directory (which is a special file with the DIR attribute set). When a cluster cannot be written to or read from, the cluster is marked as BAD and becomes unusable for the lifetime of the volume.

The size of each entry is dependent upon the format. FAT12 have 12 bit entries, FAT16, 16 bit entries, and so on.

Corruption of the FAT can result in lost cluster chains, resulting in corrupted files and/or directories. This eventually could render the file volume useless.

## Root Directory

On FAT12 and FAT16, the Root Directory immediately follows the FAT. For FAT32, the Root resides in the Data Region. Because of this, clusters are not defined for the Root Directory in FAT12 and FAT16 and the size is fixed, typically no larger than 512 entries, with the exception of very large FAT16 volumes (256MB and 512MB) . In reality, since an end marker is present, there is always one less entry, since the last entry is always the end of directory.

## Data

This is the region that hosts all of the cluster chains associated with files and directories. As mentioned, directories are a special file with the DIR attribute set.

# Non-Supported Features

Due to the tight coupling nature of the File System portion with the Data Flash, certain options are not supported.

## Partitioning

Partitioning is more suitable for movable media such as a Floppy or Hard Disk. The device driver for the Flash Device determines what part of the Flash part is accessible by the File System Driver.

## FAT32

Volumes that use FAT32 as the primary format, must be greater than 512Mbytes in size in order to be compliant with Microsoft’s standards.  FAT32 is not supported simply because there are currently no parts capable of this yet, i.e. 4 Megabit. However, FAT16 is only capable of handling volumes less than 4 Gigabytes in size and that comes with a few adjustments to the size of the ROOT directory.

## Long Filenames

The current version of the FFS only supports the traditional 8.3 naming convention (alternate naming convention when dealing with volumes that support long filenames). Because of the overhead involved and resource limitations of embedded environments and especially with FAT12 volumes, long filenames are not included with this release of the FFS.

# Flash Reliability Enhancements

Unlike a hard-drive or ram storage media, a flash device may become worn and induce errors in stored data over time. To effectively manage this and to extend the reliability of the end application, the Fusion File System has incorporated several features that greatly extend end user product reliability. These enhancements include strategic cashing of key data, wear leveling of erasable flash blocks, dynamic tracking of bad blocks and user callable error correction functions.

## Run-Time Errors

The Fusion Flash File system can dynamically handle cluster failures. This means when a section of the flash is bad or goes bad during the product life cycle the file system will dynamically mark the clusters as defective and not use those again.

## Data Caching

The Fusion File System has implemented limited caching to greatly reduce wear of flash storage media. The Fusion Flash File system will cache one data sector (usually 512 bytes) for each open file. This reduces wear for files that are frequently written to. An example may be an update of the time once a minute. The data will continue to be written to the cached data and not the flash each minute. The data will is not written until either the file is closed or new data spans across another sector. This is a cache miss. When this happens the data is stored to the flash and a new sector for the file is cached. The Fusion File system also buffers at least one data sector of the File Allocation Table (FAT). For embedded applications that have additional memory resources available, the FAT cache is user configurable. The user may specify the number of FAT data sectors to buffer. This greatly reduces the flash wear where the FAT table is located in flash.

## Wear Leveling

The Fusion Flash File System incorporates a wear-leveling algorithm to evenly distribute the wear over the entire flash device. The user sets the wear level threshold step for each level of wear. When an erasable section reaches a specified wear level the file system will mark that block as unavailable. The file system will then find a new cluster to use. When there are no longer any available sectors, the file system will reset the flash driver for a new wear level and mark all of the unavailable FAT clusters as available and then start a new wear level.

## Error Correction

The Fusion Flash File System provides user callable error correction functions. This enables the user to error encode their data and then correct it should an error occur during storage. The error correction algorithm is a Reed-Solomon Error Corrector. Reed-Solomon codes are block-based error correcting codes with a wide range of applications in digital communications and storage. Reed-Solomon codes are used to correct errors in many systems including:

- Storage devices (including tape, Compact Disk, DVD, barcodes, file systems etc)

- Wireless or mobile communications (including cellular telephones, microwave links, etc)

- Satellite communications

The Reed-Solomon encoder takes a block of digital data and adds extra "redundant" bits. Errors occur during transmission or storage for a number of reasons (for example noise or interference, scratches on a CD, etc). The Reed-Solomon decoder processes each block and attempts to correct errors and recover the original data. The number and type of errors that can be corrected depends on the characteristics of the Reed-Solomon code. For this application, two bytes out of each 32 bytes can be corrected.

Reed Solomon codes are a subset of BCH codes and are linear block codes. A Reed-Solomon code is specified as RS(n,k) with s-bit symbols.

This means that the encoder takes k data symbols of s bits each and adds parity symbols to make an n symbol codeword. There are n-k parity symbols of s bits each. A Reed-Solomon decoder can correct up to t symbols that contain errors in a codeword, where 2t = n-k.

The following diagram shows a typical Reed-Solomon codeword (this is known as a Systematic code because the data is left unchanged and the parity symbols are appended):

The version available with the Fusion File System is a RS (32,28) with 8 bit symbols. This means each 28-byte block of data is encoded into 32-byte correctable blocks. Each 32-byte block can have up to two bad bytes and still be successfully corrected.

Chapter 2
Product Perspective

# Overview of the Model

The product consists of two drivers that are tightly coupled into a single entity: File System Driver (FAT FSD) and the Data Flash Driver (specific to Atmel Data Flashes). While the lower Data Flash Driver can stand alone or be called directly, the File System Driver (FSD or FAT Driver) cannot. It is dependent upon the Flash Driver or a Driver of similarity in order to access the device the file system resides. Therefore the Flash Driver gives users a physical view of the hardware, the FSD gives them more of a logical one.

The coupling of the FSD to the FAT is done via a driver model, solely developed for this product. It’s technology is actually a potpourri of existing driver architectures integrated into a single solution. It is  stripped down for speed and portability, since this is very important for embedded software engineering. The model also allows for developers to add drivers above or below the file system.

# Device Model

The File System uses a simple model that can be ported into any environment with minimal performance degradation and memory requirements. It allows the file system to be extensible and portable to these environments with improved integration time.

There are two parts to the model: Driver Object and Device Object.

## Driver Object

The Driver Object implements the interface and operations for all devices that are similar. For example an FSD Driver for FAT. It goes to reason that the operations for read and write a FAT volume does not change from device to device, logically speaking, therefore a single Driver Object for all FAT file systems, would bind to one or more FAT devices.

| nFlags | Flags specific to the driver (currently not used) |
| --- | --- |
| nDriverNo | Identifies the Driver Class and Major Identification or Subclass (used as the prefix in all devices belonging to this driver. This part of the ID, is unique and cannot be used by any other driver in the system. |
| pDeviceList | The list of devices belonging to this driver. |
| pIoDispatch | I/O dispatcher for the driver. |
| pDriverInit | Load or initialization function for the driver. |
| pDriverUnload | Unload function for the driver. |

## Device Object

The Device Object is a data structure describing the device to which it binds. Device Objects may bind to other device objects in a chain and in the implementation of the file system, that object would be the Flash Device, which in turns binds directly to the hardware interface. This is why the Flash Driver is the physical view of the file system and the file system driver (FSD) is the logical view.

| nDeviceNo | The Major and Minor number that identifies the Device. Since the Major Number is equivalent to the Minor number of the Driver to which this device belongs, this number is unique throughout the system. |
| --- | --- |
| nFlags | Current operational and status flags for the device. |
| nRefCount | Number of open descriptors on this device (this is only used if the device is to be unloaded, with open descriptors still on it). |
| pDrvParent | The Driver this device belongs to. |
| pDevNext | The next device in the list. |
| pDevBinded | The device to which this device is binded to. If NULL, then typically this is a Physical Device Object that talks directly to the hardware or hardware API. |
| pfnDeviceInit | Initialization function for this device. |
| pDevice_Ext | The device extension area. (explained in more detail later). |

A single device object can only link to a single device object below it, but can accept multple links from above it. For the purpose of the file system, it is best to use a one to one mapping between the FSD devices and the Flash devices.

# Dispatch Interface

Access to a device is done through a dispatch interface. In reality, it is not a true dispatch as the term implies, but can be extended to one, since the data is passed via an I/O packet or object. The dispatching is simply taking the opcode of the packet and routing it to the proper handler of the driver.

## IO_DISPATCH Method

### Parameters:

| piop | Pointer to I/O Packet |
| --- | --- |

### Return status:

| iostatus | The result of the operation. |
| --- | --- |

### Prototype:

DEV_IOSTATUS (*pIoDispatch)(PDEV_IOPACKET piop);

It is advised to use the macro, IO_DISPATCH(piop), which takes a pointer to the I/O packet as an argument.

### Example:

DEV_IOSTATUS iostat;

DEV_IOPACKET iop;

iop.nIoOperation = DEVICE_IO_CONTROL;

iop.pDevice = GET_DEVICE_OBJECT(devno);

iop.nFunction =

_IOCTL_MAKECODE(

MAJOR_DEVNO( Device_GetId(_iop.pDevice) ),

_FSD_IOCTL_MOUNT, 0

);

iostat = IO_DISPATCH( &iop );

One drawback as seen here is that the Device Object of the target device is required to properly route the packet to it’s target device. The developer must either supply the device number directly or map the ID with a device mnemonic, like many other operating systems do. This mapping is optional by the user, but an example of a very simple I/O manager is included to show how devices can use these mnemonics, like “C:”, to identify them.

Also included, is an example of a File I/O API that maps system level functions that are very familiar to “C” programmers.

## I/O Packet

| pDevice | The Target Device Object the packet is destined for |
| --- | --- |
| nIoOperation | One of five I/O operations: Open, Close, Read, Write, IoControl |
| nFunction | Function of the I/O operations. This is necessary when doing IoControl. This can also be used as an attribute field for other operations, especially “Open”. |
| pDescriptor | The Descriptor for the device, which contains at the very least address information for the target. |
| pOutBuffer | The data to output to the device |
| nOutBufferLen | The length of the data to output |
| pInBuffer | The data to input from the device |
| pInBufferLen | The length of the data inputted |

More descriptive uses for the I/O Packet will be explained later, when the functionality of the FSD is explained.

# Device Driver Table

The Device Driver Table, defined within the header file devtbl.h, is the only part of the Device Object model itself that will vary from platform to platform. This Table defines the Driver Objects for the system, in this case, the FAT FSD and the Flash Port Driver. An example is provided that can be reused with little modification required to port the file system to a  new platform.

The table is order dependent and therefore it is important as to the order in which the drivers are placed in the table, since lower level drivers must initialize before higher level ones. The drivers initialized first, appear first in the table.

The contents of the Device Driver Table, in devtbl.h, are organized with the Driver Identification Section(s) first for all the device drivers in the system, followed by the table itself:

### Device Driver Identification Section

- DEVICE DRIVER ID (Major Number): A list of Ids is found in devobj.h. Only

- DRIVER EXPORTS: The three master functions for initialization, shutdown, and dispatching.

- DEVICE LIST: The list of the Device Objects Ids (can be multiple) that are aggregated by the driver. This is the Minor Number of the Device identification.

For each type of device driver in the system a group of the above will be defined.

### Driver Table

As previously mentioned, the devices go in the table FIFO, meaning they initialize from the top down.

Each entry in the Table is a Driver Object entry, defined statically. It is easiest to use the Macro, DEFINE_DEVICE_DRIVER, to define a device driver for each grouping of device driver in the system.

The file devtbl.h is very well documented and porting to a specific implementation should be painless. For the most part, very little if anything, should be done with this table, unless a new class of device drivers is added.

# DISCLAIMER

The object model used by the Flash File system is a public domain model and is therefore not covered directly by any of the copyrights pertaining to this software. The model is actually a simplistic derivation from other platform models, to provide for maximum portability and was only used to tightly integrate the File System Driver to the Flash Port Driver. It will also help for extending the file system, if other FSDs, supporting other file system types is desired, i.e. NTFS.

Chapter 3
System Requirements

The header file platform.h contains definitions that are imperative to properly port the FFS from one environment to another. The inability to handle these or define these for the specific environment will result in major modifications to the drivers to compensate and may require a larger data footprint in certain cases.

# Platform Compatibility

A table at the end of this chapter maps the functoins to their “C” equivalent. For more details on these, a “C” reference guide will be required to demonstrate their usage, since that is not the premise of this document.

## Memory Management

The file system requires several memory management functions to be provided by the target environment

- DEVOBJ_MALLOC – Allocates memory block.

- DEVOBJ_FREE – Frees (releases) a memory block.

- DEVOBJ_MEMSET - Sets buffers to a specified character.

- DEVOBJ_MEMCPY - Copies characters between buffers.

- DEVOBJ_MEMCMP - Compare characters in two buffers.

## Character Functions

The file system requires several character management functions to be provided. These are commonly found in the C-header file, ctype.h.

- DEVOBJ_ISLOWER - Returns true if the character is a particular representation of a lowercase character.

- DEVOBJ_ISUPPER - Returns true if the character is a particular representation of a uppercase character.

- DEVOBJ_TOLOWER – Converts a character to a lowercase representation.

- DEVOBJ_TOUPPER – Converts a charcter to an uppercase representation.

## String Management

The file system requires several string management functions to be provided. This is very critical since pathnames are frequently used throughout many of the calls.

- DEVOBJ_STRCPY - Copy a string.

- DEVOBJ_STRCAT – Append a string.

- DEVOBJ_STRCMP – Compare strings.

- DEVOBJ_STRNCPY - Copy n characters of one string to another.

- DEVOBJ_STRNCAT - Append n characters of a string.

- DEVOBJ_STRNCMP – Compare the first n characters of two strings.

- DEVOBJ_STRCHR - Find a character in a string.

- DEVOBJ_STRSTR – Find a substring in a string.

## Time Management

The file system requires time management functions to be provided. A system timer should be implemented that keeps some sort of base time. Because Unix time is based upon the number of seconds since 1970, some provision to handle this must be made.

- DEV_TIME – Integral type that stores the number of seconds since Jan 1st, 1970. Typically this is an unsigned long or time_t as it is known in “C”.

- DEV_LOCALTIME – A structure that stores time information in a more specific manner. The structures is typically known as a tm structure in “C” and has the following layout:

| tm_sec | Seconds after the minute - [0,59] |
| --- | --- |
| tm_min | Minutes after the hour - [0,59] |
| tm_hour | Hours since midnight - [0,23] |
| tm_mday | Day of the month - [1,31] |
| tm_mon | Months since January - [0,11] |
| tm_year | Years since 1900 |
| tm_wday | Days since Sunday – [0,6] |
| tm_yday | Days since January 1 – [0,365] |
| tm_isdst | Daylight savings time flag |

All values are integers (int or DEVOBJ_INT)

- DEVOBJ_TIME – Retrieves the current system time in number of seconds since Jan 1st, 1970.

- DEVOBJ_LOCALTIME – Converts a time value (DEVOBJ_TIME) into the LOCALTIME structure above.

## Synchronization Objects

The file system requires several synchronization object capabilities to be provided. The critical section allows only one task or thread to access it at any one time. Critical sections are typically implemented with Mutual Exclusion objects or a Semaphore with a count of “1”. The implementation of the actual object is on a per environment basis (i.e. it’s dependent of the operating system used and what type of synchronization is provided by that OS).

- CRITICAL_SECTION – Data type for a  critical section. User implementation can be as simple as a semaphore with a max count of 1 (implying mutual exclusion).

- DEVOBJ_DECLARE_CRITICAL_SECTION – Declares the CRITICAL_SECTION object, but does not initialize it.

- DEVOBJ_INIT_CRITICAL_SECTION – Initializes or defines a CRITICAL_SECTION object.

- DEVOBJ_DESTROY_CRITICAL_SECTION – Destroys an initialized CRITICAL_SECTION object.

- DEVOBJ_ENTER_CRITICAL_SECTION – Request ownership the CRITICAL_SECTION object. The task or thread is blocked if another already has ownership, until it is released (order is FCFS).

- DEVOBJ_LEAVE_CRITICAL_SECTION – Release ownership of the CRITICAL_SECTION object.

# C Language Cross Reference

## Memory Management

| Platform | C Language Equivalent |
| --- | --- |
| DEVOBJ_FREE(memblock) | void free( void *memblock ) |
| DEVOBJ_MALLOC(size) | void *malloc( size_t size ) |
| DEVOBJ_MEMCMP(buf,buf2,count) | int memcmp( const void *buf1, const void *buf2, size_t count ) |
| DEVOBJ_MEMCPY(dest,src,count) | void *memcpy( void *dest, const void *src, size_t count ) |
| DEVOBJ_MEMSET(dest,src,count) | void *memset( void *dest, int c, size_t count ) |

## Character Functions

| Platform | C Language Equivalent |
| --- | --- |
| DEVOBJ_ISLOWER(c) | int islower( int c ) |
| DEVOBJ_ISUPPER(c) | int isupper( int c ) |
| DEVOBJ_TOLOWER(c) | int tolower( int c ) |
| DEVOBJ_TOUPPER(c) | int toupper( int c ) |

## String Management

| Platform | C Language Equivalent |
| --- | --- |
| DEVOBJ_STRCAT(dest, src) | char *strcat( char *dest, const char *src ); |
| DEVOBJ_STRCHR(string, c) | char *strchr( const char *string, int c ); |
| DEVOBJ_STRCMP(string1, string2) | int strcmp( const char *string1, const char *string2 ); |
| DEVOBJ_ STRCPY (dest, src) | char *strcpy( char *dest, const char *src ); |
| DEVOBJ_STRLEN(string) | size_t strlen( const char *string ); |
| DEVOBJ_STRNCAT(dest, src, count) | char *strncat( char *dest, const char *src, size_t count ); |
| DEVOBJ_STRNCMP(str1, str2, count) | int strncmp( const char *str1, const char *str2, size_t count); |
| DEVOBJ_STRNCPY(dest, src, count) | char *strncpy( char *dest, const char *src, size_t count ); |
| DEVOBJ_STRSTR(string, strset) | char *strstr( const char *string, const char *strset ); |

## Time Management

| Platform | C Language Equivalent |
| --- | --- |
| DEVOBJ_TIME(timer) | time_t time( time_t *timer ) |
| DEVOBJ_LOCALTIME (timer,tms) | struct tm *localtime( const time_t *timer ) |

Note: for Localtime, the C-equivalent returns a pointer to the structure. This could pose problems at the device driver level, since blocking can occur which would invalidate the pointer if another task made the same call. Therefore, an example has been included with the code and in the appendix to remedy this, where localtime takes two arguments, one for the timer and one for the tm struct.

## Synchronization

There is no cross reference for Synchronization objects, since there is no provision in the language to support them. The target environment, should at worst case support Mutual Exclusion synchronization, since Critical Sections are regions of software that only allow one execution thread or task access at any one time.

The CRITICAL_SECTION object could  be nothing more than a simple semaphore implementation with a count of 1.

# Memory Requirements

Memory requirements for the FFS will vary from platform to platform, depending on the word size and the byte order (Little Endian vs. Big Endian). The feature of the Flash File system has been optimized both in data and code space, to provide maximum effeciency without sacrificing functionality.

The data resources required are described in the appendix for the major objects that are present in the FFS.

Chapter 4
File System API

The System API is the entry point into the file I/O sub-system. Although at a system level, the calls resemble those that are familiar to the ANSI-C calls commonly found in Unix operating systems. Only a subset of calls are supported, since many are redundant and for embedded products are really not required.

# File System Methods

During integration the developer can include all or a portion of these calls to meet their specific needs for their product. At a minimum, the volume operations and file operations should be included, if no directory hierarchy is required.

## Volume Specific Operations

These are methods that are device or volume-wide:

- sys_format – Formats a volume file system

- sys_mount – Mounts a file system (to a device name)

- sys_umount – Unmounts a file system

## Directory Specific Operations

These are operations specific to special files known as directories:

- sys_closedir – Closes a directory

- sys_mkdir – Creates a new subdirectory

- sys_opendir - Opens a directory

- sys_readdir - Reads and entry from a directory

- sys_rmdir – Removes a directory

## File  Specific Operations

These are operations specific to regular files:

- sys_close – Closes a file

- sys_creat – Creates a new file or truncates an existing one

- sys_open – Opens a file

- sys_read – Reads data from a file

- sys_write – Writes data to a file

## Miscellaneous (Support) Operations

These are methods that are miscellaneous, but generally are used for informational or special handling of files.

- sys_eof – Test if current file pointer is at the end of the file

- sys_lseek – Moves the file pointer in an opened file

- sys_remove – Deletes a file

- sys_rename – Renames a file

- sys_stat – Retrieves information from a file

- sys_tell – Gets the current file position in a file

# Structures

There are a few important data structures that allow the user to gather information about the file system and even perform an ls method (as shown in the appendix) so that given a pathname a listing of what is contained within that path is returned.

## DIRECT

This is a universal directory structure that is file system independent. It is filled during a call to readdir with the filename, not the full path, and an i-number, which in Unix specifies the index of a file’s inode.

| d_ino | This is the i-node number of the file, which is not supported in FAT file systems, but a value of ‘0’ regardless will indicate a deleted file or free entry. |
| --- | --- |
| d_iname | This is the string name of the file (path not inclusive). The size of this field is dependent upon the value for DIRSIZE, defined in fsd.h. |

## STAT

This structure contains information about the file and is accessed via the stat function.

| st_dev | The major and minor device numbers of the device which the file is stored on. |
| --- | --- |
| st_ino | The i-node number (i-number) of the file |
| st_mode | A set of bits encoding the type of file and the access permissions it has. |
| st_nlink | The number of hard links to the file, including the file itself. This value is always 1, since hard links are not implemented in FAT file systems. |
| st_uid | The user id of the owner of the file. In FAT this value is always 0. |
| st_gid | The group id of the owner of the file. In FAT this value is always 0. |
| st_rdev | The type of device if the file resides on a special device. This is always set to st_dev for the file system. |
| st_size | The size of the file in bytes. |
| st_atime | The last time the file was accessed (this is disabled for the file system and is only modified if the file was open for write access – prevents wear leveling problems). |
| st_mtime | The last time the file was modified |
| st_ctime | The time the file was created. |

# Types

- FILEDESC – An index number, file descriptor, that denotes an opened file. A value of –1 or UNDEFINED_FILEDESC indicates an invalid file descriptor.

- FILEPOS – An integer value representing the current offset within the file.

- DIRDESC – Synonymous with FILEDESC, except for directories.

# System Programmer’s Interface

These are the system level APIs whose purpose is to give a Linux-C compatible look and feel to standard File I/O. The biggest difference, due to linkage issues, is the return value. The return values are zero or positive on success (depending on the function) and negative for error.

Use the Macro _ISERROR(e) to determine if the return value is an error and _ERRNO(e) to get the “errno” equivalent as found in the appendices.

All of the methods are found in file_api.h and corresponding structures in fsd.h.

sys_close()

Closes a file previously opened with sys_open.

Prototype:

int sys_close( FILEDESC fd )

Input Parameters:

fd: Valid file descriptor (previously assigned by sys_open or sys_creat).

Return Values:

int; ESUCCESS = Success, <0 = Failure.

Example:

char _filname[] = “C:\\Temp\\test.txt”;
FILEDESC _fd;

_fd = sys_open( filname, _OPEN_RDWR, 0 );

if ( _ISERROR(_fd) )

{
    /*

* Handle Errors here

*/

....

}

else

{
    ....
    sys_close( _fd );

}

sys_closedir()

Closes a directory previously opened with sys_opendir.

Prototype:

int sys_closedir( DIRDESC fd )

Input Parameters:

fd: Valid file descriptor for the directory (previously assigned by sys_opendir).

Return Values:

int; ESUCCESS = Success, <0 = Failure.

Example:

char _dirname[] = “C:\\Temp”;
DIRDESC _fd;

_fd = sys_opendir( _dirname );

if ( _ISERROR(_fd) )

{
    /*

* Handle Errors here

*/

....

}

else

{
    ....
    sys_closedir( _fd );

}

sys_creat()

Creates a new file or opens and truncates an existing one.

Prototype:

FILEDESC sys_creat( char* name, int perm )

Input Parameters:

name: Name of the new file.

perm: Permissions set to a newly created file.

| _PMODE_READ | File has read access (for FAT this is always TRUE) |
| --- | --- |
| _PMODE_WRITE | File has write access |
| _PMODE_RDWR | _PMODE_READ ¦ _PMODE_WRITE |
| _PMODE_EXECUTE | Has no meaning for FAT file system |

Return Values:

FILEDESC; 0 or greater =  Valid File Descriptor (Success), <0 = Failure

Example:

char _filname[] = “C:\\Temp\\test.txt”;
FILEDESC _fd;

_fd = sys_creat( _filname, _PMODE_RDWR );

if ( _ISERROR(_fd) )

{
    ....

}

else

{
    ....
    sys_close( _fd );

}

sys_devstats()

Gets the statistics of the device, such as number of blocks and block size (used for testing drivespace). This is a proprietary function only and it’s use is to avoid the complexity of obtaining volume information by other means.

Prototype:

int sys_devstats(char* devname, PDEVSTATS stats )

Input Parameters:

devname: string name for the device (i.e. C:).

stat: Pointer to a DEVSTATS structure.

Return Values:

int; ESUCCESS = Success, <0 = Failure.

Example:

char _devname[] = “C:”;
DEVSTATS _devstats;

unsigned long _nfree;

sys_devstats( devname, &_devstats );

/*

*  The number of free blocks

*/

_nfree = _devstats.blkStats.nFreeBlocks * _devstats.blkStats.nBlockSiz;

sys_eof()

Tests for end-of-file on a valid file descriptor.

Prototype:

int sys_eof( FILEDESC fd )

Input Parameters:

fd: Valid file descriptor (previously assigned by sys_open or sys_creat).

Return Values:

int; 1 = end of file is true, 0 = end of file is not reached, <0 = Failure.

Example:

char _filname[] = “C:\\Temp\\test.txt”;
int _fd;

_fd = sys_open( _filname, _OPEN_RDONLY, 0 );

if ( _ISERROR(_fd) )

{
    ....

}

else

{
    ....

while ( !sys_eof(_fd) )

{

sys_read(...);

....

}
    sys_close( _fd );

}

sys_format()

Formats a file system on a mounted device. Care must be taken when doing a format, for if the device has already been formatted, then all data currently on that volume will be erased.

Prototype:

int sys_format( char* devname, char* volname )

Input Parameters:

devname; string name for the device (i.e. C:).

volname: string name that is used as the volume label.

Return Values:

int; ESUCCESS = Success, <0 = Failure.

Example:

See example for sys_mount.

sys_lseek()

Moves the file pointer associated with fd to a new location that is offset bytes from origin. The next operation on the file occurs at the new location.

Prototype:

FILEPOS sys_lseek( FILEDESC fd, FILEPOS offset, int origin )

Input Parameters:

fd: Valid file descriptor (previously assigned by sys_open or sys_creat).

offset: Number of bytes from origin forward or backwards.

origin: Initial position (one of three values):

_SEEK_BEG – Seek from beginning of file

_SEEK_CUR – Seek from the current position

_SEEK_END – Seek from end of file

Return Values:

FILEPOS; the offset, in bytes, of the new position from the beginning of the file, <0 = Failure.

Example:

char _filname[] = “C:\\Temp\\test.txt”;
int _fd;

_fd = sys_open( _filname, _OPEN_RDWR, 0 );

if ( _ISERROR(_fd) )

{
    ....

}

else

{

int _err = sys_lseek(_fd, 100, _SEEK_BEG );

if ( _ISERROR(_err) )

{

....
    }

....
    sys_close( _fd );

}

sys_mkdir()

Creates a new subdirectory. The path up to the new directory must exists otherwise an error will occur.

Prototype:

int sys_mkdir( char* dirname )

Input Parameters:

dirname: Fully qualified path of the directory name to create.

Return Values:

int; ESUCCESS = Success, <0 = Failure.

Example:

char _dirname[] = “C:\\Temp\\Newdir”;
int _errno;

/*

* The subdirectory Temp must exist to create Newdir below it.

*/

_errno = sys_mkdir(_dirname);

if ( _ISERROR(_errno) )

{
    ....

}

/*

* Success

*/

else

{
    ....
}

sys_mount()

Mounts a filesystem so that file I/O operations can take place. If using the I/O Manager and have it configured correctly, then the user can set auto-mounting, as in Unix, of file systems at initialization time.

NOTE: The sys_mount and sys_unmount functions are the only two functions that return an actual error code, i.e. errno.

Prototype:

int sys_mount( char* devname )

Input Parameters:

devname: String name of the device.

Return Values:

int; ESUCCESS = Success, <0 = Failure.

Example:

char _devname[] = “C:”;
int _errno;

_errno = sys_mount( _devname );

/*

*  Format error?

*/

if ( _ISERROR(_errno ) )

{

/*

*  Failure is sometimes due to the file system not being

*  formatted

*/

if ( _errno == ENOEXEC )

{

_errno = sys_format( _devname )

/*

* Critical FILE SYSTEM error?

*/

if ( _ISERROR(_errno) )

{

return _ERRNO(_errno);

}

....

}

else

{

return _ERRNO(_errno);

}

}

sys_open()

Opens the file specified by the fully qualified pathname, name. If the filename is a directory (see sys_opendir), then the directory must exist and if it does, only opens it for read-only access.

Prototype:

FILEDESC sys_close( char* name, int flags, int perm)

Input Parameters:

name: Fully qualified pathname of the file.

flags: Type of operations allowed on file.

| _OPEN_RDONLY | Open the file for reading only (default) |
| --- | --- |
| _OPEN_WRONLY | Open the file for writing only |
| _OPEN_RDWR | Open the file for reading and writing |
| _OPEN_APPEND | Append to the file when writing rather than at the beginning |
| _OPEN_CREAT | Create the file if it does not exist (perm should be used when this mode is specified) |
| _OPEN_TRUNC | Truncates the file to zero length if opened for writing |
| _OPEN_EXCL | Return an error if the file is to be created and already exists |
| _OPEN_DELETE | Deletes a file on close (use sys_remove instead) |

perm: Permissions set to a newly created file (ignored if file already exists), see sys_creat.

Return Values:

FILEDESC; 0 or greater =  Valid File Descriptor (Success), <0 = Failure.

Example:

char _filname[] = “C:\\Temp\\test.txt”;
int _fd;

_fd = sys_open( filname, _OPEN_CREATE | _OPEN_EXCL, _PMODE_RDWR );

if ( _ISERROR(_fd) )

{
    /*

* Handle Errors here

*/

....

}

else

{
    ....

sys_close( _fd );

}

sys_opendir()

Opens the file specified by the fully qualified pathname, dirname.

NOTE: Directories can only be open as read-only.

Prototype:

DIRDESC sys_opendir( char* dirname )

Input Parameters:

char*: Fully qualified pathname of the directory.

Return Values:

DIRDESC; 0 or greater =  Valid File Descriptor (Success), <0 = Failure.

Example:

char _dirname[] = “C:\\Temp”;
int _fd;

/*

* Open the Temp Directory

*/

_fd = sys_opendir( _dirname );

if ( _ISERROR(_fd) )

{
    ....

}

else

{
    ....
    sys_closedir( _fd );

}

sys_read()

Closes a file previously opened with sys_open.

Prototype:

int sys_read( FILEDESC fd, void* buf, unsigned int count )

Input Parameters:

fd: Valid file descriptor (previously assigned by sys_open or sys_creat).

buf: Buffer to read into.

count: Maximum number of bytes to read.

Return Values:

int; The number of bytes read or EFAIL if an error occurred. If the number of bytes read is less than count, then an EOF condition was probably encounter (see sys_eof).

Example:

char _filname[] = “C:\\Temp\\test.txt”;
int _fd;

char buf[128];

_fd = sys_open( _filname, _OPEN_RDWR, 0 );

if ( _ISERROR(_fd) )

{
    ....

}

else

{

int _nread;

while ( (_nread = sys_read( _fd, buf, 128 )) > 0 )

{

....

}

if ( _ISERROR(_nread) )

{

/*

*  Handle Errors here

*/

}
    ....
    sys_close( _fd );

}

sys_readdir()

Reads a single entry from a directory, previously opened with sys_opendir.

Prototype:

int sys_readdir( DIRDESC fd, PDIRECT dir )

Input Parameters:

fd: Valid file descriptor (previously assigned by sys_opendir).

dir: Pointer to a DIRECT structure, which is defined as follows:

| d_ino | The i-node number (i-number) of the file. If “0”, then this entry is an unused or free entry. |
| --- | --- |
| d_name[DIRSIZE] | Name of the file up to DIRSIZE characters |

Return Values:

int; ESUCCESS = Success, <0 = Failure.

Example:

char _dirname[] = “C:\\Temp”;
int _fd;

_fd = sys_opendir( _dirname );

if ( _ISERROR(_fd) )

{
    ....

}

else

{

DIRECT _dir;

int _errno;

while( (_errno = sys_readdir( _fd, &_dir )) == ESUCCESS )

{

/*

*  Free or Unused entry?

*/

if ( _dir.d_ino == 0 )

{

continue;

}

....
    } 
    ....
    sys_closedir( _fd );

}

sys_remove()

Deletes a file from the file volume. Directories can also be removed using this function as long as they are empty.

sys_unlink performs the same operation.

Prototype:

int sys_remove( char* name )

Input Parameters:

name: Fully qualified pathname of the file.

Return Values:

int; ESUCCESS = Success, EFAIL = Unsuccessful.

Example:

char _filname[] = “C:\\Temp\\test.txt”;
int _errno;

_errno = sys_remove( _filname );

if ( _ISERROR(_errno) )

{
    /*

*  Errors are handled here

*/

....
}

sys_rename()

Closes a file previously opened with sys_open.

Prototype:

int sys_rename( char* oldname, char* newname )

Input Parameters:

oldname: Fully qualified pathname of the file.

newname: Name for the new file (not fully qualified).

Return Values:

int; ESUCCESS = Success, <0 = Unsuccessful.

Example:

char _filname[] = “C:\\Temp\\test.txt”;

char _newname[] = “test.old”
int _errno;

_errno = sys_rename( _filname, _newname );

if ( _ISERROR(_errno ) )

{
    /*

*  Errors are handled here

*/

....
}

else

{

int _fd;

DEVOBJ_STRCPY( _filname, “C:\\Temp\\” );

DEVOBJ_STRCAT( _filname, _newname );

_fd = sys_open( _filname, _OPEN_RDWR, 0 );

if ( _ISERROR(_fd) )

{
        ....

}

else

{
        ....
        sys_close( _fd );

}

}

sys_rewinddir()

Rewinds the directory to the first entry. Since directories can only be opened as read-only and their size, indeterminate on some file systems, seeking on them is only allowed in this manner.

Prototype:

int sys_rewinddir( FILEDESC fd )

Input Parameters:

fd: Valid file descriptor.

Return Values:

int; ESUCCESS = Success, <0 = Unsuccessful.

Example:

char _dirname[] = “C:\\Temp”;
int _fd;

/*

* Open the TEMP Directory

*/

_fd = sys_opendir( _dirname );

if ( _ISERROR(_fd) )

{
    ....

}

else

{

DIRECT _dir;

while ( !_ISERROR( sys_readdir( _fd, &_dir ) )

{

....

}

/*

*  Rewind to first entry in the directory

*/

rewinddir( _fd );

....
    sys_closedir( _fd );

}

sys_rmdir()

Deletes a directory.

Prototype:

int sys_rmdir( char* dirname )

Input Parameters:

dirname: Path of directory to be removed.

Return Values:

int; ESUCCESS = Success, EFAIL = Unsuccessful.

Example:

char _dirname[] = “C:\\Temp”;
int _errno;

/*

* The subdirectory Temp must exist to create Newdir below it.

*/

_errno = sys_rmdir(_dirname);

if ( _ISERROR(_errno) )

{
    ....

}

sys_stat()

Get status information on a file or directory.

Prototype:

int sys_stat( char* name, PSTAT pstat )

Input Parameters:

name: Pathname of the file.

pstat: Pointer to structure of type STAT

Return Values:

int; ESUCCESS = Success, EFAIL = Unsuccessful.

Example:

char _filname[] = “C:\\Temp\\test.txt”;

STAT _stat;

int _errno;

_errno = sys_stat( filname, &_stat );

if ( !_ISERROR(_errno) )

{

/*

* Is FILE Writable?

*/

if ( _stat.st_mode & _SMODE_IWRITE )

{

....

}
    ....

}

sys_tell()

Get the current file position.

Prototype:

FILEPOS sys_tell( FILEDESC fd )

Input Parameters:

fd: Valid file descriptor (previously assigned by sys_open or sys_creat).

Return Values:

FILEPOS; Current position of the file descriptor or EFAIL if the file descriptor is invalid.

Example:

char _filname[] = “C:\\Temp\\test.txt”;
int _fd;

char buf[128];

_fd = sys_open( _filname, _OPEN_RDWR, 0 );

if ( _ISERROR(_fd) )

{
    ....

}

else

{

int _nread;

while ( (_nread = sys_read( _fd, buf, 128 )) > 0 )

{

....

}

if ( _ISERROR(_nread) )

{

/*

*  Handle Errors here

*/

}

/*

*  Determine size of file based on file position

*/

else

{

FILEPOS _fp = sys_tell( _fd );

....

}
    ....
    sys_close( _fd );

}

sys_umount()

Unmounts a filesystem, rendering it unusable until it is mounted again. The filesystem must have been previously mounted with the sys_mount function or an error will be returned.

Prototype:

int sys_umount( char* devname )

Input Parameters:

devname: String name of the device.

Return Values:

int; ESUCCESS = Success, errno = Unsuccessful.

Example:

char _devname[] = “C:”;
int _errno;

_errno = sys_mount( _devname );

/*

*  Format error?

*/

if ( _ISERROR(_errno ) )

{

/*

*  Test for ERROR Type (see sys_mount)

*/

....

}

/*

*  If execution gets here, then the system is mounted and formatted

*/

....

sys_umount( _devname );

sys_unlink()

Unlinks or removes a file from the file system. This function is the same as the sys_remove function described earlier.

Prototype:

int sys_unlink( char* name )

Input Parameters:

name: Fully qualified pathname of the file.

Return Values:

int; ESUCCESS = Success, EFAIL = Unsuccessful.

Example:

char _filname[] = “C:\\Temp\\test.txt”;
int _errno;

_errno = sys_unlink( _filname );

if ( _ISERROR(_errno ) )

{
    /*

*  Errors are handled here

*/

}

sys_write()

Closes a file previously opened with sys_open.

Prototype:

int sys_write( FILEDESC fd, void* buf, unsigned int count )

Input Parameters:

fd: Valid file descriptor (previously assigned by sys_open or sys_creat).

buf: Buffer to write from.

count: Maximum number of bytes to write.

Return Values:

int; The number of bytes written or EFAIL if an error occurred.

Example:

char _filname[] = “C:\\Temp\\test.txt”;
int _fd;

_fd = sys_open( _filname, _OPEN_RDWR | _OPEN_TRUNC, 0 );

if ( _ISERROR(_fd) )

{
    ....

}

else

{

int _nwritten;

char buf[128];

do

{

/*

*  Initialize the buffer withs some data

*/

....

_nwritten = sys_write( _fd, buf, 128 )

}

while ( !_ISERROR(_nwritten) );

if ( _ISERROR(_nwritten) )

{

/*

*  Handle Errors here

*/

}
    ....
    sys_close( _fd );

}

Chapter 5
I/O Manager

The I/O manager included with the Flash File System manages the mapping between application and driver layers. The I/O manager released here is also responsible for keeping synchronization between open files on a device and the tasks that access them.

The version included enforces mutual exclusion only as a more complex scheme would require much more software and more interaction between the I/O Manager and the File System Driver.

# I/O Table

The I/O Table is an extension of the Driver table but it specifically targets the device objects and not the drivers and only those devices that have character names assigned to them. Basically it’s intention is for all devices that are FSDs.

- Map the logical device names to their Device Ids (Major and Minor Number) so that calls can be dispatched to the appropriate device.

- Manage the File Control Block Table for each device

## IOEntry

Each member of the I/O is defined as follows:

| szDevname | The name of the device up to DEV_STRLEN characters. |
| --- | --- |
| nDevId | The device number (Major and Minor number) |
| bAuto | Indicates whether the device is automatically mounted |
| tblFCB | The table of FCBs. The length of the table is FCB_TBLLEN and since a Hash Table is used, it’s best that this be equivalent to a prime number to reduce the number of collisions. |

The device name allows higher level and even system level functions to access the device as a string name. For example, file systems can be named C, D, E, AUX, etc. By default the device delimiter is the old MS-DOS “:”, but this can be altered by developers by redefining DEVNAME_DELIMETER, found in iomgr.h.

With this table, the I/O Manager can do a sequential search, if more than one file system exists, and map the device mnemonic (name) to it’s ID which is used to aid in dispatching calls to the device.

NOTE: The table is not dynamic and is therefore NOT updated due to mounting/unmounting of file systems.

# Initialization

There are two calls, not mentioned in the file system API that are essential in startup and shutdown of the FFS.

- sys_startup

- sys_shutdown

It is VERY important that prior to using the FFS, that sys_startup be invoked, otherwise failures, perhaps catastrophic may occur. The I/O manager is responsible for mounting the file systems to their devices and setting up all the linkage between itself and the devices. Semaphores are also initialized that help to protect accessing of files and descriptors during this time.

Basically the file system should be initialized sometime during or even after the target System initializes itself (OS Initialization) and should be shutdown prior to OS Shutdown.

Sys_main()

{
    /* OS STARTUP */

....

sys_startup();

/* RUN THE OS */

....

sys_shutdown();

....

/* OS SHUTDOWN */

}

sys_shutdown should be invoked, whenever the system powers down, or even goes into standby mode, as this will commit all open file descriptors, closing them, preventing data loss. Descriptors left open and not handle properly could result in a fragmented file system that contains lost clusters that can never be allocated.

# File Control Block

The last entry in the IOEntry block is a table that contains an array of pointers to File Control Blocks. This table maintains the number of open descriptors from the file system’s point of view (to be explained shortly). The File Control Block, FCB, is simply a structure that although transparent to the client, is very important to the driver, since the File Object is part of this block.

There is a single FCB table or list per device, therefore for each file system there is an FCB table it. When a request to allocate a FCB, via a File Descriptor (returned to the user as an integer), the I/O Manager verifies the pathname of the request by checking the table for all existing file descriptors allocated to that file.

## Implementation

For this version, FCBs are mutually exclusive and therefore, if an entry in the table matches the pathname of the user request, then an error is returned along with an invalid file descriptor. Therefore the operation, an open request, fails. The problem is, however, that there are many ways that a pathname can be derived that points to the same file and by hashing this string, multiple FCB entries for the same file could exist in the table.

For example:

C:\temp\..\test\.\files\..\files\thisfile.dat = C:\test\files\thisfile.dat

Now typically no one in their right mind would pass such a pathname, as shown on the left, to the file system. However, in situations where things are running in an automated fashion and builds a filename based on criteria without verification (since it may not know the valid character set of a pathname), such a condition could occur. The file system must be able to handle such conditions, regardless. Therefore, the first thing the I/O manager does, after verifying the device (“C:”), is to normalize the pathname part of the string before running it through the hash function, h(x). By doing this, both versions of the previous file name will evaluate to the same index or bucket in the table.

An “open” hash table is used by the I/O Manager to store the FCB entries. Hashing the FCBs allow for quicker access when using the full pathname as the key into the table. An open table is used, since collisions can occur due to the possibly infinite number of potential members. An open hash table uses a linked list for each bucket in the table. Therefore if a collision occurs, then the FCB is appended as the last element in the list.

The previous diagram shows the memory layout of the FCB Table with B buckets (suggested prime number).

Chapter 6
File System Driver

The File System Driver, FSD, is any driver that manages a file volume on a device. There are numerous FSDs available such as VFAT, FAT, NTFS, etc. The FAT, specifically FAT12 and FAT16, will be discussed in detail here, although the operations typically port from one file system to the next with no changes to the arguments.

# FSD Objects

## FAT Device Extension

Each Device Object has an extension area that only the Driver for those devices knows the layout of. For the FAT driver, the device extension is defined as follows:

| FAT_DataClusters | The count of total data clusters |
| --- | --- |
| FAT_FreeClusters | The count of free data clusters |
| FAT_FirstFreeCluster | The index of the first free cluster |
| FAT_RootDirSectors | The number of root directory sectors |
| FAT_FirstRootDirSector | The index of the start of the root directory |
| FAT_Format | The format type of the volume, FAT12 or FAT16 |
| FAT_Eof | The End-of-File marker for the volume, since this varies between format types. |
| FAT_CacheSector | The current sector of the File Allocation Table currently cached. |
| FAT_Cache | The cache buffer size |
| FAT_CriticalSection | The critical section object guarding the FAT |
| FAT_BootSector | The boot sector information for the volume (this follows byte for byte the boot record stored. See appendix). |

Some of the information within the extension is included to reduce the amount of time to calculate it based upon other fields. Since these calculations would be performed quite often, it speeds access time with minimal effect to memory resources.

## File Object

The file object is a descriptor used by the FSD to describe information about the access of the file. Not all devices ever used may require a file object, but for the FSD it is mandatory. There is a single file object for a single open instance of a file. Typically file objects exist one for one with a descriptor table defined higher up in the I/O system architecture. These descriptors may actually index another structure that contains both an instance of the file object and another point to an FCB, which allows synchronization between files and directories in the system as a higher level.

| pDeviceObj | Pointer to the device object the file object references |
| --- | --- |
| nFlags | Status flags of the file object |
| nPos | Absolute position within the file |
| pFileExt | File extension area |
| nBufferLen | Length of the file cache |
| pBuffer | File buffer or cache. |

## FAT File Extension

The File extension for FAT Volumes is as follows:

| theEntry | Directory entry for the file (directories are special entries) |
| --- | --- |
| theParentEntry | The parent entry for the file (directory in which it resides) |
| nDirSector | Sector the directory entry for the file exists |
| nDirOffset | Offset within the sector the directory entry for the file exists |
| nCluster | Current cluster index (corresponds to the file position) |
| nOffset | Current offset within the cluster (the sector can be computed given cluster and offset). |

# Dispatch Operations

The I/O Dispatch functionality was discussed previously and within the packet was a field that contained the I/O operation and function to be performed. Typically the function code is used for I/O Control operations only.

The following is the “5” I/O Operations, which are placed in the nOperation field of the I/O Packet for the FSD and how they are setup prior to dispatching and what the return code and information is. Fields not listed below are not used by the Operation.

NOTE: All packets must fill the pointer to the Device Object as this is imperative to routing the request.

## DEVICE_IO_OPEN

Opens a file for reading and/or writing.

| pDescriptor | Pointer to a pre-allocated (unitialized) File Object |
| --- | --- |
| pOutBuffer | Name of the FILE to open, path inclusive |
| nFunction | Open flags and permissions packed into a single word |

## DEVICE_IO_CLOSE

Closes and opened file.

| pDescriptor | Pointer to the open File Object |
| --- | --- |

## DEVICE_IO_READ

Inputs data from a file.

| pDescriptor | Pointer to the open File Object |
| --- | --- |
| pInBuffer | Pointer to the buffer to read into |
| pInBufferLen | Size of the buffer, indicating the number of characters to read |

## DEVICE_IO_WRITE

Outputs data to a file

| pDescriptor | Pointer to the open File Object |
| --- | --- |
| pOutBuffer | Pointer to the buffer to write from |
| nOutBufferLen | Size of the buffer, indicating the number of characters to write |

## DEVICE_IO_CONTROL

Performs a specific I/O operation on the device, based on the function code passed. The following are the function codes, nFunction, supported by the File System:

### mount

Mounts a volume, allowing access to the file system. A mount must be the first I/O Control operation performed on a file system before access to files and directories are permitted.

No parameters required

### unmount

Unmounts a mounted file volume. The unmount must the last I/O Control operation performed

| No parameters required | No parameters required |
| --- | --- |

### format

Formats a file system onto a mounted volume.

| pOutBuffer | Pointer to volume label |
| --- | --- |
| nOutBufferLen | Length of the volume label |

### lseek

Seek to a new position in an open file.

| pDescriptor | Pointer to the open File Object |
| --- | --- |
| pOutBuffer | Pointer to the offset to seek |
| nOutBufferLen | Set to indicate the Origin to seek from |
| pInBuffer | Pointer to place position after seek |

### rename

Renames a file and/or directory.

| pDescriptor | Pointer to a File Object |
| --- | --- |
| pOutBuffer | Pointer to the new name |
| nOutBufferLen | String length of the new name |
| pInBuffer | Pointer to the old name |
| pInBufferLen | String length of the old name |

### mkdir

Create a subdirectory

| pDescriptor | Pointer to a File Object |
| --- | --- |
| pOutBuffer | Pathname of the directory to create |
| pOutBufferLen | Length of the pathname |

### rmdir

Remove a subdirectory

| pDescriptor | Pointer to a File Object |
| --- | --- |
| pOutBuffer | Pathname of the directory to remove |
| pOutBufferLen | Length of the pathname |

### readdir

Reads a directory entry.

| pDescriptor | Pointer to the open File Object |
| --- | --- |
| pInBuffer | Pointer to a DIRECT structure |
| nInBufferLen | sizeof(DIRECT) |

### filestat

Gets information on a file

| pDescriptor | Pointer to the open File Object |
| --- | --- |
| pOutBuffer | Pointer to the pathname of file |
| nOutBufferLen | String length of the pathname |
| pInBuffer | Pointer to a STAT function |
| pInBufferLen | sizeof(STAT) |

Chapter 7
Integration and Usage Tips

# ROOT Directory Management

One tip that will help wear-leveling of the FLASH part near the front, is to reduce amount of write cycles to the ROOT directory. It is always best to create a subdirectory farm in the root, then build the tree off of that. The reason for this is simply that the ROOT directory in FAT12 and FAT16 is at a fixed location. It  cannot be moved and once it goes bad, the file system becomes unstable and possibly unusable. Subdirectories on the other hand can be shifted between different locations as wear leveling conditions are detected to load level the part and extend the life of it as long as possible.

# FAT Determination

The source code is currently equipped to autodetect if the target part should be formatted for FAT12 and FAT16. There are rules to this if the system is to be compatible with other FAT File Systems.

| FAT Type | Sizemin | Sizemax | Clustersmin | Clustersmax |
| --- | --- | --- | --- | --- |
| FAT12 | 0 | < 4.1MB | 0 | 4084 |
| FAT16 | 4.1 MB | 2.0 GB | 4085 | 65524 |
| FAT32 | 32.5 MB | > 32 GB | 65525 |  |

NOTE: FAT32 is not supported with this version as it typically not the format of choice unless the part is at least 512 MB in size (this is 4 Gigabit Flash).

| Atmel Part | Density | FATtype | Clusterssize | Sectorscluster |
| --- | --- | --- | --- | --- |
| AT45DB011B | 1 MBit | FAT12 | 512 bytes | 1 |
| AT45DB021B | 2 MBit | FAT12 | 512 bytes | 1 |
| AT45DB041B | 4 MBit | FAT12 | 1024 bytes | 2 |
| AT45DB081B | 8 MBit | FAT12 | 1024 bytes | 2 |
| AT45DB16B | 16 Mbit | FAT12 | 1024 bytes | 2 |
| AT45DB321B | 32 Mbit | FAT12 | 2048 bytes | 4 |
| AT45DB642 | 64 Mbit | FAT16 | 1024 bytes | 2 |
| AT45DB1282 | 128 MBit | FAT16 | 1024 bytes | 2 |
| AT45DB2562 | 256 MBit | FAT16 | 2048 bytes | 4 |

## Initialization

When the file system is mounted to the device, it queries the target device as to information about the minimum block size it supports on read/write and the number of these blocks available. The total part size is determined on that and is used in one of the two lookup tables including in the software. As seen in the first table preceeding this section, devices up to and including 4.0 MBytes in size are always FAT12 and anything over that and up to 512 Mbytes is FAT16.

The tables, located at the top of the file fat_vol.c, are based upon 512 sector sizes, and by Microsoft’s definition cannot be used for any sectors larger, but arithmetic shifting based on sector sizes of 1024, 2048 will work just fine. Typically for sector sizes greater than 512, a device should be at the very least 4.1 Mbytes in size, but this is not a mandatory requirement.

Appendix A
Miscellaneous

# Compatible errno codes

These codes are compatible with those defined by Unix, although not all these are returned by the Flash File System.

| Error Code | Definition |
| --- | --- |
| EPERM | Operation not permitted |
| ENOENT | No such file or directory |
| ESRCH | No such process |
| EINTR | Interrupted system call |
| EIO | I/O error (read or write) |
| ENXIO | No such device or address |
| E2BIG | Argument list is too long |
| ENOEXEC | Exec format error |
| EBADF | Bad file number |
| ECHILD | No child processes |
| EAGAIN | Try again |
| ENOMEM | Out of memory |
| EACCES | Permission denied |
| EFAULT | Bad address |
| ENOTBLK | Block device required |
| EBUSY | Device or resource busy |
| EEXIST | File exists |
| EXDEV | Cross-device link |
| ENODEV | No such device |
| ENOTDIR | Not a directory |
| EISDIR | Is a directory |
| EINVAL | Invalid argument |
| ENFILE | File table overflow |
| EMFILE | Too many open files |
| ENOTTY | Not a typewriter (obsolete) |
| ETXTBSY | Text file busy |
| EFBIG | File is too large |
| ENOSPC | Device is out of space |
| ESPIPE | Illegal seek |
| EROFS | Read-only file system |
| EMLINK | Too many links |
| EPIPE | Broken pipe |
| EDOM | Math argument out of domain of function |
| ERANGE | Math result not representable |
| EDEADLK | Resource deadlock would occur |
| ENAMETOOLONG | File name too long |
| ENOLCK | No record locks available |
| ENOSYS | Function not implemented |
| ENOTEMPTY | Directory not empty |
| ENOSTR | Device not a stream |
| ENODATA | No data available |
| ETIME | Timer expired |
| ENOMEDIUM | No medium found |
| EMEDIUMTYPE | Wrong medium type |

# DEVSTATS

This structure is actually a union, since a device can either be a block or character type device. Because it is a union the nType field overlaps with the nType field of both the structure types for Block or Character.

For the File System, both the Flash Driver and FAT File System Driver are Block Devices.

| nType | Identifies the device type, either block or character. |
| --- | --- |
| blkStats | Block device structure |
| chrStats | Character device structure |

## BLOCK_DEVSTATS

| nType | Identifies the device type, either block or character. |
| --- | --- |
| nClass | Device classification |
| nBlockSize | Size of blocks (in bytes) for the device |
| nTotBlocks | Total number of blocks in device |
| nFreeBlocks | Total number of available blocks |

## CHAR_DEVSTATS

| nType | Identifies the device type (. |
| --- | --- |
| nClass | Device classification |
| nTotalChars | Total number or maximum number of characters |
| nAvailChars | Number of current available characters |

# Exception Handling

There is builtin exception handling to the drivers and the functionality on the working of these can be found in the file, platform.h.

There are two implementation methods used based upon what resources the development platform contains:

- METHOD 1: SETJMP / LONGJMP Pair

Traditionally, the ANSI-C method of doing exception handling by method of non-local gotos. If there is no support non-local gotos in the development platform, then the 2nd method must be used.

- METHOD 2: GOTO

This violates methods of traditional structured programming practices, but since speed is relevant in this case, sometimes loop nesting prevent a more optimal solution.

## FUNCTIONALITY

| _TRY |  |
| --- | --- |
| _CATCH(e) |  |
| _AND_CATCH(e) |  |
| _END_CATCH |  |
| _THROW(e) |  |
| _THROW_NULL() |  |

## DEFINED EXCEPTIONS

| Exception_Unhandled |  |
| --- | --- |
| Exception_None |  |
| Exception_Default |  |
| Exception_Memory |  |
| Exception_Dispatch |  |
| Exception_File |  |
| Exception_Device |  |

## Example

The following example is just a basic usage of the Exception Handling macros implemented throughout the File System Driver.

int main()

{

_TRY

{

....

_THROW(Exception_Memory);

....

}

_CATCH(Exception_File)

{

/* Handle File specific errors here */

}

_AND_CATCH(Exception_Memory)

{

/* Memory specific exceptions are handled here */

}

_END_CATCH

return 0;

}

Appendix B
Memory Requirements

- The following tables are the memory requirements necessary for each object allocated in the system and the maximum number of each object.

| - Object | - Maximum Allocated | - Persistence | - Memory Required (in BYTES) |
| --- | --- | --- | --- |
| - DRIVER_OBJ | - 2 total (one for Atmel Flash devices and one for FAT volume devices) | - Until driver unload | - 48 (24 for each one) |
| - ATMEL_FLASH_DEVOBJ | - 1 per flash device | - Until device unload | - 28 |
| - FAT_DEVOBJ | - 1 per FAT volume | - Until device unload | - 188 + (2 * Sector Size) |
| - DEV_IOPACKET | - 1 per request | - Until I/O completion | - 32 |
| - IOENTRY | - 1 per FAT volume | - Until device unload | - 40 |
| - File Descriptors | - FILEDESC_MAX (defined in file_api.c) | - Continuous | - 4 |
| - FCB | - Maximum number of descriptors allowed | - Until descriptor releases | - 64 |
| - FAT_FILEOBJ | - Maximum number of descriptors allowed | - Until file closes or descriptor releases | - 84 + Sector Size |

- NOTE: Pointers and WORD size  are assumed 32 bits.

The table does not account for any buffers locally used to read and write data, only internal caching.

So assume a file system that allows a maximum of 16 files open on it at any one time and an I/O request is being performed on each open file. Also assume that the sector size is 512 (which by default for this file system is the case), then the total number of bytes required is as follows:

| - Object | - Allocated | - Memory Required |
| --- | --- | --- |
| - DRIVER_OBJ | - 2 | - 48 |
| - ATMEL_FLASH_DEVOBJ | - 1 | - 28 |
| - FAT_DEVOBJ | - 1 | - 1212 |
| - DEV_IOPACKET | - 16 | - 512 |
| - IOENTRY | - 1 | - 40 |
| - File Descriptors | - 16 | - 64 |
| - FCB | - 16 | - 1024 |
| - FAT_FILEOBJ | - 16 | - 9536 |
| - TOTAL |  | - 12464 or 12.17 KB |

- NOTE: This does not take into consideration of internal I/O requests nor the buffers allocated for each I/O packet.

Appendix C
Examples

# Doing a UNIX “ls” (directory listing)

This function was written and tested on a Windows platform emulating the Flash File System environment. There is a non-printable version that puts the entire directory into a string provided in support.c.

#define LIST_LINELEN        80

#define LIST_CRLF           "\x0d\x0a"

DEV_LONG ListDir(

DEV_STRING strPath,

DEV_STRING* pstrListing

)

{

DIRDESC _hDir;

DIRECT _direct;

DEV_STRING _strFile;

DEV_STRING _strCur;

DEV_LONG _nEntries = 0;

/*

*  Debug

*/

DEVOBJ_ASSERT( strPath != 0 );

DEVOBJ_ASSERT( pstrListing != 0 );

/*

*  Open the directory

*/

do

{

_hDir = sys_opendir( strPath );

if ( _ISERROR(_hDir) )

{

break;

}

/*

*  Count the number of Entries

*/

while ( sys_readdir( _hDir, &_direct ) == ESUCCESS )

{

if ( _direct.d_ino != 0 )

{

_nEntries++;

}

}

if ( _nEntries == 0 )

{

*pstrListing = 0;

break;

}

sys_rewinddir(_hDir);

/*

*  Allocate the buffer

*/

*pstrListing =

(DEV_STRING)DEVOBJ_MALLOC(LIST_LINELEN * _nEntries );

if ( *pstrListing == 0 )

{

break;

}

*pstrListing[0] = '\0';

_strCur = *pstrListing;

/*

*  Now for each entry

*/

_strFile = DEVOBJ_MALLOC( DEVOBJ_STRLEN(strPath) + DIRSIZE );

if ( _strFile == 0 )

{

break;

}

while ( sys_readdir( _hDir, &_direct ) == ESUCCESS )

{

STAT _stat;

/*

*  Valid Entry?

*/

if ( _direct.d_ino != 0 )

{

DEVOBJ_STRCPY( _strFile, strPath );

DEVOBJ_STRCAT( _strFile, "\\" );

DEVOBJ_STRCAT( _strFile, _direct.d_name );

/*

*  Format the DATE and TIME

*/

if ( !_ISERROR( sys_stat( _strFile, &_stat ) ) )

{

register DEV_LONG _nLen;

DEV_LOCALTIME* _pLocalTime;

DEVOBJ_LOCALTIME( &_stat.st_mtime, _pLocalTime );

/*

*  Concatenate Date

*/

DEVOBJ_STRCPY( _strCur, "00" );

DEVOBJ_LTOA( _pLocalTime->tm_mon + 1, _strFile, 10 );

_nLen = DEVOBJ_STRLEN(_strFile);

if ( _nLen < 2 )

{

*( ++_strCur ) = *_strFile;

}

else

{

DEVOBJ_STRCPY( _strCur, _strFile );

}

_strCur += _nLen;

*_strCur++ = '/';

DEVOBJ_STRCPY( _strCur, "00" );

DEVOBJ_LTOA( _pLocalTime->tm_mday, _strFile, 10 );

_nLen = DEVOBJ_STRLEN(_strFile);

if ( _nLen < 2 )

{

*( ++_strCur ) = *_strFile;

}

else

{

DEVOBJ_STRCPY( _strCur, _strFile );

}

_strCur += _nLen;

*_strCur++ = '/';

DEVOBJ_LTOA(

_pLocalTime->tm_year + 1900,

_strFile, 10

);

DEVOBJ_STRCPY( _strCur, _strFile );

_strCur += DEVOBJ_STRLEN(_strFile );

DEVOBJ_STRCPY( _strCur, "  " );

_strCur += 2;

/*

*  Concatenate Time

*/

DEVOBJ_STRCPY( _strCur, "00" );

DEVOBJ_LTOA( _pLocalTime->tm_hour + 1, _strFile, 10 );

_nLen = DEVOBJ_STRLEN(_strFile);

if ( _nLen < 2 )

{

*( ++_strCur ) = *_strFile;

}

else

{

DEVOBJ_STRCPY( _strCur, _strFile );

}

_strCur += _nLen;

*_strCur++ = ':';

DEVOBJ_STRCPY( _strCur, "00" );

DEVOBJ_LTOA( _pLocalTime->tm_min + 1, _strFile, 10 );

_nLen = DEVOBJ_STRLEN(_strFile);

if ( _nLen < 2 )

{

*( ++_strCur ) = *_strFile;

}

else

{

DEVOBJ_STRCPY( _strCur, _strFile );

}

_strCur += _nLen;

*_strCur++ = ':';

DEVOBJ_STRCPY( _strCur, "00" );

DEVOBJ_LTOA( _pLocalTime->tm_min + 1, _strFile, 10 );

_nLen = DEVOBJ_STRLEN(_strFile);

if ( _nLen < 2 )

{

*( ++_strCur ) = *_strFile;

}

else

{

DEVOBJ_STRCPY( _strCur, _strFile );

}

_strCur += _nLen;

*_strCur++ = ':';

DEVOBJ_STRCPY( _strCur, "00" );

DEVOBJ_LTOA( _pLocalTime->tm_sec + 1, _strFile, 10 );

_nLen = DEVOBJ_STRLEN(_strFile);

if ( _nLen < 2 )

{

*( ++_strCur ) = *_strFile;

}

else

{

DEVOBJ_STRCPY( _strCur, _strFile );

}

_strCur += _nLen;

/*

*  Type of FILE and Size

*/

if ( (_stat.st_mode & _SMODE_IFMT) == _SMODE_IFDIR )

{

DEVOBJ_STRCPY( _strCur, "     <DIR>          " );

_strCur += 20;

}

else

{

DEVOBJ_LTOA( _stat.st_size, _strFile, 10 );

DEVOBJ_STRCPY( _strCur, "                    " );

_nLen = DEVOBJ_STRLEN(_strFile);

_strCur += ( 19 - _nLen );

DEVOBJ_STRNCPY( _strCur, _strFile, _nLen++ );

_strCur += _nLen;

}

}

/*

*  Finally the FILE NAME

*/

DEVOBJ_STRCPY( _strCur, _direct.d_name );

_strCur += DEVOBJ_STRLEN( _direct.d_name );

DEVOBJ_STRCPY( _strCur, LIST_CRLF );

_strCur += DEVOBJ_STRLEN( LIST_CRLF );

}

}

DEVOBJ_FREE( _strFile );

}

while(0);

/*

*  And CLOSE OUT

*/

sys_close( _hDir );

/*

*  Return the total number of valid entries

*/

return _nEntries;

}

# localtime

One of the problems with the C-library version of localtime is that, like strtok, it uses a static variable that returns a pointer to the caller:

struct tm* localtime( time_t* timer )

Synchronization is a problem in this regard, unless a guard in the form of a mutex is wrapped around the call, allowing only one task access at anyone time. The problem there is, the client or caller would have to provide the mechanism by acquiring the mutex, then releasing it when finished, otherwise a second thread could inadvertantly call the function, corrupting the previous callers information, although minimally.

Included in the file support.c (for show only) is an example of implementing the localtime function as the following more thread-safe version:

DEV_LOCALTIME* LocalTime(

DEV_TIME nTime,

DEV_LOCALTIME ptmTime

)

/*

*  Constants

*/

#define _SECONDSPERMINUTE       60L

#define _MINUTESPERHOUR         60L

#define _HOURSPERDAY            24L

#define _SECONDSPERHOUR         ( _MINUTESPERHOUR * _SECONDSPERMINUTE )

#define _SECONDSPERDAY          ( _SECONDSPERHOUR * _HOURSPERDAY )

#define _DAYSPERWEEK            7L

#define _DAYSPERYEAR            365L

#define _MONTHSPERYEAR          12L

enum _MONTHSOFYEAR {

eJanuary = 0,

eFebruary,

eMarch,

eApril,

eMay,

eJune,

eJuly,

eAugust,

eSeptember,

eOctober,

eNovember,

eDecember,

eTotalMonths

};

enum _DAYSOFWEEK {

eSunday = 0,

eMonday,

eTuesday,

eWednesday,

eThursday,

eFriday,

eSaturday,

eTotalDays

};

DEV_LOCALTIME* LocalTime(

DEV_TIME nTimer,

DEV_LOCALTIME* ptmTime

)

{

static DEV_UINT s_nMonthDay[] = {

31, 28, 31, 30, 31, 30, 31, 31, 30, 31, 30, 31

};

register DEV_INT _nIdx;

/*

*  Debug

*/

DEVOBJ_ASSERT( ptmTime != 0 );

/*

*  The time is based on the UNIX time, which is seconds since

*  January 1, 1970

*

*      Get seconds offset within minute (covert to minutes)

*      Get minutes offset within hour (convert to hours)

*      Get hours offset within day (convert to days)

*/

ptmTime->tm_sec = DEVOBJ_REM(nTimer,_SECONDSPERMINUTE);

nTimer /= _SECONDSPERMINUTE;

ptmTime->tm_min = DEVOBJ_REM(nTimer,_MINUTESPERHOUR);

nTimer /= _MINUTESPERHOUR;

ptmTime->tm_hour = DEVOBJ_REM(nTimer,_HOURSPERDAY);

nTimer /= _HOURSPERDAY;

/*

*  nTimer is now set to the number of days (calculate the day of the

*  week, since Jan 1, 1970 was on a "Thursday (day 4)"

*/

ptmTime->tm_wday = DEVOBJ_REM(( nTimer + eThursday ), _DAYSPERWEEK);

/*

*  Now for the DATE Part, adjust to days and remember to calculate

*  leap years.

*/

ptmTime->tm_year = nTimer / _DAYSPERYEAR;

nTimer = DEVOBJ_REM(nTimer,_DAYSPERYEAR);

/*

*  NOW adjust the time based on Leap Year information

*/

nTimer -= (( ptmTime->tm_year + 2 ) / 4 );

/*

*  Is current year a Leap Year?

*

*      Adjust year since it is year since 1900 (i.e. add 70)

*      and set the YEAR Day

*/

ptmTime->tm_year += 70;

ptmTime->tm_yday = nTimer;

if ( DEVOBJ_REM(ptmTime->tm_year,4) == 0 )

{

s_nMonthDay[ eFebruary ] = 29;

}

else

{

s_nMonthDay[ eFebruary ] = 28;

}

/*

*  Calculate Month/Day

*/

_nIdx = 0;

while (1)

{

if ( nTimer < (DEV_TIME)s_nMonthDay[_nIdx] )

{

ptmTime->tm_mday = nTimer + 1;

break;

}

nTimer -= s_nMonthDay[_nIdx];

ptmTime->tm_mon;

/*

*  Next Month

*/

_nIdx++;

}

/*

*  The Month is wherever the Index currently is (Will never reach

*  _MONTHSPERYEAR or shouldn't)

*/

ptmTime->tm_mon = _nIdx;

ptmTime->tm_isdst = -1;

return ptmTime;

}

Index

A

Appendix A Miscellaneous	61

Appendix B Memory Requirements	66

Appendix C Examples	68

B

BLOCK_DEVSTATS	63

Boot Sector	4

C

C Language Cross Reference	19

CHAR_DEVSTATS	64

Character Functions	16, 19

Compatible errno codes	61

D

Data Caching	7

Device Driver	14

Device Driver Table	14

Device Model	10

Device Object	11

DEVICE_IO_CLOSE	55

DEVICE_IO_CONTROL	56

DEVICE_IO_OPEN	55

DEVICE_IO_READ	56

DEVICE_IO_WRITE	56

DEVSTATS	63

Direct	24

Directory Operations	23

Disclaimer	15

Dispatch Operations	55

Driver Object	10

Driver Table	14

E

Error Correction	8

Example localtime	72

Example UNIX is	68

Exception Defines	65

Exception Example	65

Exception FUNCTIONALITY	64

Exception Handling	64

F

FAT	5

Fat Control Block	51

FAT Determination	59

Fat Device Extension	53

Fat File Extension	54

FAT Overview	4

FAT32	6

File Object	54

File Operations	23

File System API	22

File System Driver	53

filestat	58

File-System Methods	22

Flash Reliability Enhancements	6

format	57

FSD Objects	53

I

I/O Dispatch Interface	12

I/O Manager	49

I/O model Overview	9

I/O packet	13

I/O Table	49

Implementation	51

Index	76

Initialization	50, 60

Integration and Usage Tips	59

Introduction	4

IO_DISPATCH Method	12

IOEntry	49

L

lseek	57

M

Memory Management	16, 19

Memory Requirements	21

Miscellaneous Operations	23

mkdir	57

mount	56

P

Partitioning	6

Platform Compatibility	16

Product Perspective	9

R

readdir	58

rename	57

rmdir	58

Root	5

ROOT	59

Run-Time Errors	6

S

Stat	24

String Management	17, 20

Structures	24

Synchronization	21

Synchronization Objects	18

sys_close	26

sys_closedir	27

sys_creat	28

sys_devstats	29

sys_eof	30

sys_format	31

sys_lseek	32

sys_mkdir	33

sys_mount	34

sys_open	35

sys_opendir	37

sys_read	38

sys_readdir	39

sys_remove	40

sys_rename	41

sys_rewinddir	42

sys_rmdir	43

sys_stat	44

sys_tell	45

sys_unlink	47

sys_unmount	46

sys_write	48

System Programmers Interface	26

System Requirements	16

T

Time Management	17, 20

Types	25

U

unmount	57

V

Volume Operations	22

W

Wear Leveling	7